| A Billing of Quantities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sr No | Item Name |  | Make |  | Description |  |  | Unit | Quantity |
| 1 | SOLAR MODULE |  | PAHAL/RAYZON |  | TOPCon (Tunnel Oxide Passivated Contact) bi-facial PV modules are next-generation high-efficiency solar panels that generate power from both front and rear surfaces. These modules utilize advanced n-type cell technology, offering higher efficiency, better low-light performance, and lower degradation rates compared to traditional PERC modules. With bifacial gain ranging from 5% to 25% depending on site conditions, TOPCon modules deliver superior energy output and long-term reliability—ideal for commercial, industrial, and utility-scale installations. |  |  | Nos | 10 |
| 2 | INVERTER |  | SUNGROW |  | HIGH YIELD - Lower startup & wider MPPT voltage EASY AND USER FRIENDLY - Fast and easy commissioning via iSolarCloud App SAFE AND RELIABLE Build-in Type II DC & AC SPD C5 anti-corrosion & IP65 Remote firmware updates |  |  | Nos | 1 |  |
| 3 | MODULE MOUNTING STRUCTURE |  | HINDUSTAR |  | 60 Micrones Hot Dip Galvanized (GI) pipes with 2MM Thickness are steel pipes coated with a layer of zinc through a hot-dip process, providing superior corrosion resistance and long-term durability. These pipes are commonly used in solar mounting structures due to their strength, weather resistance, and maintenance-free performance. Ideal for outdoor solar installations, GI pipes ensure structural stability and extended service life in diverse environmental conditions. |  |  | KG | 350 |  |
| 4 | MAIN AC DISTRIBUTION BOX |  | MCB - SCHNEIDER / LK (L&T) MCCB - SCHNEIDER / LK (L&T) CONTACTOR - SCHNEIDER / LK (L&T) NVR - STANDERD SPP - GIC SPD - FINDER MMF METER - SECURE ENCLOSER - (Note - ABS used till 50-60 kW) |  | The ACDB is a critical component in the solar power system that safely manages and distributes AC power from the inverter to the load or grid. It is equipped with: MCB (Miniature Circuit Breaker) and MCCB (Molded Case Circuit Breaker) for circuit protection Contactor for automatic switching control NVR (No-Volt Release) and SPP (Single Phase Preventer) for protection against phase failure and voltage loss SPD (Surge Protection Device) for protection against voltage surges MMF Meter (Multifunction Meter) for real-time monitoring of voltage, current, power, and energy This fully integrated ACDB ensures system safety, reliable operation, and accurate energy monitoring in solar installations. |  |  | NOS | 1 |  |
| 5 | DC DISTRIBUTION BOX |  | DC MCB - SCHNEIDER / LK (L&T) SPD - FINDER |  | The DC Distribution Box (DCDB) is designed to protect and control the DC output from solar PV modules. It is equipped with: DC MCB (Miniature Circuit Breaker) for protection against overcurrent and short circuits SPD (Surge Protection Device) to protect against voltage surges and lightning-induced spikes This robust and weatherproof DCDB ensures the safe and reliable operation of the solar power system, enhancing overall system durability. |  |  | NOS | 1 |  |
| 6 | ESE LIGHTING ARRESTOR |  | Conventional Lightning Arrestor |  | Provides conical protection MOC. Copper with required Spikes for Building and Chimneys Protection |  |  | NOS | 1 |  |
| 7 | EARTHING ROAD |  | ERTH - COPPER BONDED SOLID ELECTRODE |  | Copper Bonded Earth rods made from high tensile low carbon steel by molecularly bonding 99.9% pure copper with a minimum coating thickness of 250 micron System is Corrosion resistant & rated for high fault currents Moisture booster chemical bags provided for low earth resistance Low maintenance on site |  |  | NOS | 3 |  |
| 8 | DC CABLE |  | APAR / POLYCAB / RR KABEL / HAVELLS |  | Supply and laying of DC 1 Core x 4.0 sqmm copper flexible cable (Apar / Polycab / RR Kabel / Havells make), XLPE insulated, UV and weather resistant, suitable for interconnection of solar PV modules and DC circuits. Ensures high conductivity, low voltage drop, and long-term outdoor durability. |  |  | Meter | 80 |  |
| 9 | AC CABLE |  | APAR / POLYCAB / RR KABEL / HAVELLS |  | Supply and laying of AC 2 Core x 4.0 sqmm copper flexible cable (Apar / Polycab / RR Kabel / Havells make), PVC insulated and sheathed, suitable for AC power distribution in solar systems. Offers high conductivity, flexibility, and reliable performance under varying electrical and environmental conditions. |  |  | Meter | 40 |  |
| 10 | AC CABLE- 2 |  | APAR / POLYCAB / RR KABEL / HAVELLS |  | Supply and laying of AC 3.5 Core x 50 sqmm Aluminium Armoured Cable (Apar / Polycab / RR Kabel / Havells make), XLPE insulated, PVC sheathed, suitable for LT power distribution in solar power systems. Designed for outdoor and underground installation, offering high current-carrying capacity, mechanical durability, and thermal stability under continuous load conditions. |  |  | Meter | 0 |  |
| 11 | EARTHING CABLE / STRIP |  | INDOCAB |  | Supply and laying of Earth 1 Core x 4.0 sqmm Copper Cable, PVC insulated, single-core, suitable for effective grounding of solar PV structures and equipment. Provides excellent conductivity, mechanical strength, and reliable fault current dissipation for enhanced system safety. |  |  | KG/METER | 100 |  |
| 12 | LA CABLE / STRIP |  | KANBERY / FLEXGUARD / GEONOM |  | Supply and laying of Earth 1 Core x 16 sqmm Aluminium Flexible Cable, PVC insulated, suitable for earthing of solar structures and LT panels. Provides cost-effective grounding solution with adequate conductivity and flexibility for reliable fault current dissipation and system safety. |  |  | KG/METER | 50 |  |
| 13 | CABLE TRAY |  | FRP |  | Supply and installation of FRP Cable Tray, size 50mm (W) x 50mm (H) x 3mm (T) – FRP make, corrosion-resistant, UV-stabilized, and lightweight. Suitable for safe and durable routing of DC/AC cables in solar power installations under harsh outdoor conditions. |  |  | MTR | 0 |  |
| 14 | WALK WAY |  | FRP |  | Supply and installation of FRP Solar Walkway, size 272mm (W) x 3660mm (L) x 25mm (H), with mesh size 38mm x 38mm. Made from corrosion-resistant, UV-stabilized fiber-reinforced plastic, suitable for safe and durable access pathways between solar module rows on rooftops or ground-mounted structures. |  |  | MTR | 0 |  |
| 15 | FOUNDATION BLOCK |  | HTSS SCOPE |  | RCC Foundation Block – 300x300x300 mm, M25 grade concrete |  |  | NOS | 6 |  |
| 16 | Fire & Safety |  |  |  | Fire Extinguisher & Sign Bord Marking |  |  | Nos | As Per Required |  |
| 17 | INSTALLATION & COMMISSIONING |  |  |  | Hi – Tech Solar Solution Scope |  |  |  |  |  |